# MATLAB Assignment # 3

# By: Jonathan Lopez

# ECE 2010

# Section B03; Mailbox # 207

# Due 11/10 9PM

# MATLAB PROBLEM 3 	ECE2010	B19

# Matrix Operations with MATLAB. Nodal Analysis

Due Submission Online– Sunday Nov 10th by 9:00 PM

Preamble

In our third MATLAB exercise, we will learn how to solve systems of linear equations in MATLAB. A collection of fast and accurate matrix solvers is one huge advantage of MATLAB. This is the core of the MATLAB engine. The matrix solvers are based on an open-source LAPACK package (http://www.netlib.org/lapack/#_presentation), supported by both MathWorks, Inc. (creator of MATLAB) and Intel Corporation. These matrix solvers can handle very large matrices (actually unlimited in size) and are important for many practical problems.

Your task #1:

Explain what the following commands (operations with matrices) "mean" by summarizing the net result of the command. Give the explanation for the end result only (omit some repeated steps).

A = eye(4, 4)

A = ones(4, 4) + zeros(4, 4)

A = diag([2 4 6])

A = magic(4)

A = magic(4), B = A'

A = magic(4), B = sort(A)

- A = magic(4), d = det(A)

A = magic(4), B = triu(A)

a = [2 4 6], b = [1 3 5], A = kron(a, b')

a = [1:7]; b = [1:7]; A = kron(a, b'); surf(A)

| 1 | Creates a 4x4 identity matrix |
| --- | --- |
| 2 | Creates a matrix of all 1s this case 4x4 and adds it to an array of all zeros |
| 3 | 3x3 diagonal matrix with the number 2, 4, and 6 going diagonally |
| 4 | Return a matrix of 4x4 using number of 1 to 4^2. It has equal row and column sums. |
| 5 | Sets B = A’ but uses different numbers |
| 6 | Sorts the matrix from low to high higher number on the bottom of the matrix |
| 7 | Finds the determinate |
| 8 | Make an upper triangle of the numbers rest are zeros |
| 9 | Makes 2 matrix’s of two sets respectively and kron returns the Kronecker tensor product of matrices of b prime |
| 10 | Makes a surface plot of A of the above |

Nodal analysis

We already know some standard methods of solving electric circuits such as the source transformation theorem, and Thévenin/Norton theorems. There exists however another general method, which allows us to solve any linear circuit. This method is known as nodal analysis and is described in Section 4.1 of the class text. Its “heart” is the solution of a system of linear equations for unknown voltages. Let’s assume that we need to determine current I in the circuit shown in the following figure.

Fig. 1. A resistive circuit.

The nodal analysis operates with the absolute values of the node voltages in the circuit with respect to ground reference. It may be divided into a number of distinct steps:

1. A ground reference needs to be assigned first – a node where the voltage is set to 0 V. To this end, we ground the negative terminal of the voltage power supply.

2. Next, we select nontrivial (also called non-reference) nodes for which we do not know the voltages. These are nodes 1 and 2 in Fig. 1. All other nodes are eliminated from the analysis since the voltages there are already known (0 V or 10 V).

3. We label absolute unknown node voltages versus ground reference as  - see Fig. 1.

4. We label currents for every nontrivial node, assuming that all currents are outflowing – see Fig. 1.

5. Next, the KCL is written for every nontrivial node. We express the currents as the difference of two absolute voltages: the voltage at the beginning of the current arrow (voltage at the master node) minus the voltage at the end of the current arrow (voltage at any other node) and then divide this difference by the appropriate resistance. Hence, we arrive at a system of linear equations for the nodal voltages. Currents are no longer involved.

6. After the resulting system of linear equations is solved, all circuit parameters are determined as necessary.

Let’s apply this method. KCL for node 1 gives

(1)

KCL for node 2 yields

(2)

After trivial transformations, Eqs. (1) and (2), which form a system of linear equations, become

(3)

The corresponding solution obtained via Gaussian elimination is . Then, by Ohm’s law, .

The solution for a system of linear equations in MATLAB is given by a left matrix divide. The following MATLAB block

clear all;

A = [3,   -1;

-1,   3];    %   system matrix (square)

B =  [10; 10];    %   excitation vector (column)

V = A\B           %   solution of the system of linear equations - node voltages

is used to obtain the solution of the system of linear equations (3). This block solves the system of linear equations   for the column vector of unknown node voltages . Try it out, please (copy and paste the text into a new MATLAB scrip, then run this MATLAB script)!

An extension for the case of multiple unknown node voltages is straightforward. A similar procedure is used in scientific calculators. MATLAB could solve linear systems with many thousands or even millions of unknowns using built-in direct or iterative matrix solvers.

Your task #2:

1. Please make sure that you understand the short MATLAB script typed above.

2. For a hypothetic circuit with three unknown node voltages, solve the following system of linear equations in MATLAB:

(4)

Using the above MATLAB script as a starting point, create your own MATLAB script to accomplish this task. Do not type in the MATLAB command window!

Text of your MATLAB script clear all; A = [4, 1, -5; 6, 3, 2; 7, 1, 5]; % system matrix (square) B = [10; 20; 30]; % excitation vector (column) V = A\B

Values of : 3.9640 V1 -1.8018 V2 0.8108 V3

Your task #3:

You are asked to solve a circuit shown in Fig. 2 (find unknown current I) using the nodal analysis.

Fig. 2. A resistive circuit to be solved using nodal analysis.

- Introduce ground and label unknown node voltages

- Write a system of nodal equations for unknown voltages

- Solve this system in MATLAB.

Text of your MATLAB script clear all; A = [7, -3; -3, 7]; % system matrix (square) B = [30; 30]; % excitation vector (column) V = A\B

Values of unknown voltages: V = 7.5000 V1 7.5000 V2

Your report (a PDF document uploaded to CANVAS by 9 PM on this coming Sunday) should include:

- A title page with your name, section number and ECE mailbox number;

- This completed document with all the results.

There is a MATLAB help session this coming Sunday from 7 PM to 9 PM in AK317a (the lab room).