# MATLAB Assignment # 6

# By: Jonathan Lopez

# ECE 2010

# Section B03; Mailbox # 207

# Due 12/5 @ 9PM

# MATLAB PROBLEM 6 	ECE2010	B19

Due Date – Thursday, December 5th by 9:00 PM

# Preamble

In our sixth MATLAB assignment, we will

- investigate an amplifier with positive feedback and solve the relevant examples using MATLAB;

- learn about powerful MATLAB function subplot;

- learn about basic noise signal modeling in MATLAB.

Please include a cover page on your completed worksheet.

# Task #1 Modeling a comparator amplifier as a zero-level detector using MATLAB

1.1 Comparator amplifier

Fig. 1 shows two circuits. On the left, we see an open-loop (inverting) comparator amplifier studied in class.

Fig. 1 Simple comparator (left) and an amplifier circuit with the positive feedback – the inverting Schmitt trigger (right). Note that in Lecture #14 the same Schmitt trigger circuit was drawn a bit differently: with the positive feedback loop on top. Both circuit forms are equivalent, and both of them are widely used.

The inverting comparator amplifier in Fig. 1 operates in the following way:

(1a)

(1b)

where  is the supply voltage. We will assume that = 12 V.

1.2 Comparator amplifier as a zero-level detector

The comparator amplifier from the left of Fig. 1 could be used as a zero-level detector. The following MATLAB script describes the comparator operation for a noisy sinusoidal signal with an amplitude of 1V and a frequency of 100 Hz, over the time interval from 0 to 0.1 s. The added white voltage noise is created by MATLAB function rand. This function needs to be properly initialized and scaled to generate the required amount of noise. One way of doing so is illustrated in the script.

clear all

t = [0:2e-4:0.1];       %   time array, s

f = 100;                 %   frequency, Hz

Vin = 1*sin(2*pi*f*t) + 1.0*(rand(1, length(t))-0.5);   %   input voltage signal plus some noise

Vout = zeros(1, length(t));         %   initialize output voltage, V

F    = figure;                      %   initialize MATLAB figure

for m = 2:length(t)

if Vin(m-1)-0<0

Vout(m) = +12.0;     %   power rail

else

Vout(m) = -12.0;     %   power rail

end

%   graphics

plot(t, Vin, 'b', t, Vout, 'r');

axis([min(t) max(t) -14 +14]);    %   define axes

grid on;

drawnow

end

Note: This MATLAB script actually solves the circuit in time domain: it finds the next value of at step m using the previous value of at step . Step m corresponds to the current time moment  whereas step corresponds to a previous time moment .

Your task:

- Inspect the MATLAB script given above and make sure that you understand how it works including the time-stepping algorithm.

- Create your own blank MATLAB script, and copy and paste the given MATLAB code into that script.

- Run your own MATLAB script and observe the results. You might want to run it several times. The output will change slightly every time since the noise pattern is random.

- Properly label figure axes; introduce figure caption (legend).

- How do we add random noise to a harmonic signal at 100 Hz? What function does the job? Please describe what this function does.

Answer to question 5: The rand function in MATLAB can do it. It randomly distributes numbers in an array and combined with the sine function it could make the random noise at 100Hz

- What problem do you see with the comparator amplifier? Is it a “good” zero-level detector when noise is present? Is it a “good” zero-level detector when noise is absent? Document your answer (present the comparator output figures for the signal with and without noise, respectively, including labeled axes and an appropriate title). Keep the script.

Answers to question 6: It is not a good zero level detect with noise since it changes too much with the noise. There are double lines on the graph which mean that the graph changes very quickly according to the noise and could trigger when you don’t want it to. The noise can be from the light’s phones and other electronic devices.

Comparator output for a signal without noise (properly label figure axes; introduce figure caption): It is a good zero level detector when no noise is apresent it changes when it is supoosed to and mot at random times like the one with noise

Comparator output for a signal with noise (properly label figure axes; introduce figure caption):

# Task #2 A positive-feedback amplifier (“comparator with memory”) as a zero-level detector using MATLAB

Fig. 1-right shows the most famous positive-feedback amplifier circuit called the Schmitt trigger, studied in class. It looks like an inverting amplifier, but the inverting and non-inverting inputs are reversed. The idea behind this circuit is to compare the input voltage not with 0 V as in Eqs. (1), but rather with in Fig. 1. From the dynamic point of view,  may be regarded as the previous value of the output voltage times , i.e.

(2)

In other words, the circuit has a memory of its previous state and behaves accordingly. This is a concept which is extensively studied in more advanced ECE classes. Eventually, it will lead us to digital memory cells of different topologies. It will be assumed here that

(3)

in the present example. Therefore,

(4)

Your task:

- Modify your existing MATLAB script in order to model the Schmitt trigger instead of the comparator. In fact, it may be enough to slightly modify only one line of the code(!):

if Vin(m-1)-0<0

- Run your MATLAB script, including noise generation, and observe the results. You might want to run it several times; the output will slightly change every time since the noise pattern is random.

- Is the previous problem solved? Document your answer.

Change(s) made in the code: clear all t = [0:2e-4:0.1]; % time array, s f = 100; % frequency, Hz Vin = 1*sin(2*pi*f*t)+ 1.0*(rand(1, length(t))-0.5); % input voltage signal plus some noise Vout = zeros(1, length(t)); % initialize output voltage, V F = figure; % initialize MATLAB figure for m = 2:length(t) if Vin(m-1)-(1/20)*Vout(m-1)<0 Vout(m) = +12.0; % power rail else Vout(m) = -12.0; % power rail end % graphics plot(t, Vin, 'b', t, Vout, 'r'); axis([min(t) max(t) -14 +14]); % define axes grid on; xlabel('Time (s)') ylabel(' Voltage (V)') legend('Vin','Vout') drawnow end

Schmitt trigger as a zero-level detector: the resulting figure (with random noise):

# Task #3 SUBPLOT function of MATLAB

The MATLAB function subplot allows us to use the same figure window for multiple plots. Its splits the window into a number of smaller plots, each of which can be treated independently. To learn how this function works type help subplot in the MATLAB command window and hit ENTER.

Your task:

- Using function subplot modify your MATLAB script in order to plot the responses of the comparator and the Schmitt trigger simultaneously, on the same figure.

Hint:

Introduce two output voltages, for each circuit separately. For example, a block

subplot(2,1,1)

plot(t, Vin, 'b', t, Vout1, 'r');

axis([min(t) max(t) -14 +14]);    %   define axes

grid on;

subplot(2,1,2)

plot(t, Vin, 'b', t, Vout2, 'r');

axis([min(t) max(t) -14 +14]);    %   define axes

grid on;

plots two such voltages dynamically, within the MATLAB for loop. Do not forget to use the command drawnow, which updates the figure at every time step.

Remember to include a legend on each axis that has more than one line plotted.  Hint: You can call subplot() to switch between axes at will.

Figure with the comparator output and the Schmitt trigger output displayed using MATLAB function subplot: ****Note the schmitt trigger Vout(1) is on top